## Sencore 1089 API

##### Version 1.23

### Table of Contents

### History

### Overview

#### Enumerations

#### Functions Declarations

### Control Functions

### Initialize1089			Initialize the hardware

### Uninitialize1089		Un-initialize the hardware

GetRevisions1089	Retrieve the revision of the hardware device, the DLL, and the device driver.

### ExecutePlay1089		Stream video

### ExecuteStop1089		Stop the streaming of video

### GetProgramCount1089	Retrieve the currently streaming video program count and program numbers

### GetProgramCountFile1089	Retrieve the a program count and program numbers from a file

Set8VSBAdjustChannel1089	Set the output channel for 8-VSB

OnProgressSetup1089		Setup the OnProgress callback function.

OnStatusSetup1089		Setup the OnStatus callback function.

GetRevisionXilinx1089	Retrieve the revision of the Xilinx load (firmware).

ExecutePlaySeamless1089	Play files seamlessly.

RestampStream1089	Set the restamping mode (enable hardware restamping)

Board1089_Set8VsbVct	Set the 8VSB Freq. and the VCT channel

Board1089_SetVct		Set the VCT major, minor, and mode

GetCurrentlyPlayingFile1089	Get the current filename

### Callbacks

### OnProgress1089	The callback function for the current percent (point) where the video is streaming data from.

### OnStatus1089	The callback function for a message being passed back to the app.

### History:

### 06/25/2002 – version 0.00 	- Created

### 04/12/2007 – version 1.23 	– Removed obsolete functions

### Overview:

The API requires only 4 functions for streaming a HDTV file with a program index of 0 as the default. The functions are Initialize1089, Uninitialize1089, ExecutePlay1089, and ExecuteStop1089.

The 8-VSB output channels can be adjusted by calling Set8VSBAdjustChannel1089.  The 8-VSB default channel is 14.

File streaming control is handled by three functions ExecutePlay1089, ExecuteStop1089, and ExecutePause1089.

Program Count is adjusted/retrieved by GetProgramCount1089, and GetProgramCountFile1089.  ExecutePlay1089 can also adjust which program is selected when the video starts a play.

Callbacks into the app are setup by OnProgressSetup1089 and OnStatusSetup1089.

Multiple cards can be installed into a system.  The cards are usually enumerated in PCI slot order which puts them in numerical order starting with device 0 closest to the processor.  When initializing each card (Initialize1089) the device id (the passed in parameter) for each card must be unique.   Normally the device id would start with 0, then 1, and so on.  This id value must be passed when calling each function to control the card desired.

Registry:

File Stream:

Location: <Local Machine>\Software\Sencore\Stream\Settings

FileInfo: 1 = save file information, 0 = no save – A 1 will generate .info_X files where X is the program index.

TSID:  The number for the PSI and PSIP TSID numbers for insertion. Default: 0x1001 for the first board 0x1003, then 0x1005, …

1089 Board:

Location: <Local Machine>\Software\Sencore\Board1089\Settings

DMA_ENABLED: 1 = DMA transfers, 0 = PIO transfers – Default:DMA

8VSB Channel X: The default starting 8VSB channel, where X is the board starting with zero.  Default: 14

DEBUG: 1 = Save debug logging in a file in the current directory, 0 = no save

Buffer_Size_MBytes: Internal Producer/Consumer buffer, Minimum is 2 Mbytes

MAX_CARDS: Maximum boards in the system, Default is 6

INSERT_SYNC: Debug use only

Types of operation:

There are two types of operation: non-seamless and seamless.

Note:  Initialize the application as described in the “Overview:” section.  Setup a callback function and use function OnStatusSetup1089 to link it to the library.

Non-Seamless:

Use function ExecutePlay1089, with the appropriate parameters, to start the 1089 card playing. Handle the message STATUS_PLAY_COMPLETE.  Once this message is received issue an ExecuteStop1089, then use function ExecutePlay1089 with a new file or the same file.

Seamless:

Use ExecutePlaySeamless1089, with the appropriate parameters, to start the 1089 card playing. Handle the message STATUS_PLAY_START.  Once this message is received issue then use function ExecutePlaySeamless1089 with a new file or the same file.  Note:  If ExecutePlaySeamless1089 is not used at this time the currently playing file will loop upon itself (seamlessly) until the ExecutePlaySeamless1089 function is called or of course the ExecuteStop1089 function is called.

Enumerations:

## typedef enum

## {

## STATUS_BASE_8VSB = 0x10001000,

## STATUS_BASE_DLL = 0x10002000,

## STATUS_BASE_MAIN = 0x10003000

## } ENUM_STATUS_BASE;

typedef enum

{

STATUS_FAILURE = -1,

STATUS_SUCCESS,

// Initialization process fail

STATUS_INITIALIZE_ERROR_DLL = STATUS_BASE_DLL,

// If Start >= End

STATUS_PLAY_RANGE_INVALID,

STATUS_BANK0_READY = STATUS_BASE_MAIN,

STATUS_BANK1_READY, // Reserved

STATUS_RECORD_COMPLETE, // Reserved

ERROR_BANK_CONFLICT, // Reserved

ERROR_FILE_NOT_OPEN, // Reserved

ERROR_FILE_TOO_SMALL, // Reserved

ERROR_FILE_READ_NO, // Reserved

ERROR_FILE_WRITE_NO, // Reserved

ERROR_INTERRUPT, // Reserved

ERROR_MEMORY_ALLOCATION, // Reserved

ERROR_FILE_PLAY_LIMIT, // Reserved

// STATUS - Indicates a file has starting loading into system memory

STATUS_PLAY_START,

STATUS_OCX_STOP_VIDEO, // Reserved

STATUS_PAUSE_PLAY, // Reserved

ERROR_INVALID_ASPECT_RATIO, // Reserved

STATUS_VIDEO_PAUSE, // Reserved

ERROR_PTS_NOT_FOUND, // Reserved

ERROR_PCR_NOT_FOUND, // Reserved

// STATUS - Indicates a file had ended loading into system memory

STATUS_PLAY_COMPLETE,

// STATUS - Indicates that 'Video Locked Interrupt' has occurred

// (New Mpeg Data arrival)

STATUS_PROGRAM_UPDATE,

STATUS_RESTART_QPSK, // Reserved

STATUS_STREAM_UPDATE, // Reserved

STATUS_NO_SIGNAL, // Reserved

STATUS_STARTUP_VIDEO, // Reserved

STATUS_MONITOR_RESTART, // Reserved

STATUS_STREAM_START, // Reserved

STATUS_STREAM_STARTED, // Reserved

STATUS_STREAM_PSIP, // Reserved

STATUS_STREAM_STOPPED, // Reserved

STATUS_VIDEO_STARTED, // Reserved

STATUS_VIDEO_STOPPED, // Reserved

// same as STATUS_PROGRAM_UPDATE

STATUS_VIDEO_LOCKED,

// Non-seamless data monitoring indicating a video or audio change

STATUS_NEW_PSIP,

STATUS_SYNC_READY, // Reserved

ERROR_VIDEO_START, // Reserved

ERROR_STREAM_CHANNEL, // Reserved

// Socket setup and ready to connect

STATUS_SOCKET_LISTEN,

STATUS_SOCKET_WAIT, // Reserved

// A data underflow has happened pausing playback on the card,

// but allowing the system buffers to fill again.

STATUS_BUFFER_EMPTY_PAUSED,

// System buffers are full continuing playback

STATUS_BUFFER_FULL_PLAYING,

// No programs found in "File Program Information Search"

ERROR_NO_PROGRAMS,

// Starting playback error trying to open or add file to queue

ERROR_SEAMLESS_MODULE,

// No PIDs found in "File Program Information Search"

ERROR_NO_PIDS,

// Not getting proper amount of data from "Seamless" module

ERROR_TRANSFER_SIZE,

// Invalid 19392658 bitrate

ERROR_INVALID_ATSC_BITRATE,

STATUS_END,

} ENUM_STATUS;

## typedef enum

## {

## BOARD_TYPE_300_A,

## BOARD_TYPE_300_B,

## BOARD_TYPE_400_A

## } ENUM_BOARD_TYPE;

DLL Interface with VB

The functions can be used with a VB application by adding the VB declarations (See next section) to a “*.bas” file (or see the sample application file “modGlobal.bas”).

DLL Interface with C/C++

Add the file “Exports1089.h” to the project for compiling the exported DLL functions.

## Add the file “Board1089.lib” to the project for linking the exported DLL functions.

## Control Functions:

# Initialize1089                                                                                                                      _

Function:	Initializes the 1089 hardware device.

Usage:		long Initialize1089( long lngDevice );

Parameters:	lngDevice – The device to control.

Note:	The system can have multiple 1089 Cards; which the first device is 0, the second is 1, and so on.  To control each subsequent function just place this device number in the lngDevice parameter for each funtion.  This function has to be called to initialize each device.

Return Value:	STATUS_FAILURE – Already initialized

STATUS_SUCCESS

STATUS_INITIALIZE_ERROR_DLL – Hardware initialization error

Example:	 Initialize1089 ( 0 );

# Uninitialize1089                                                                                                                  _

Function:	Uninitializes the 1089 hardware device.

Usage:		long Uninitialize1089(long lngDevice);

Parameters:	lngDevice – The device to control

Return Value:	STATUS_FAILURE – Hardware not initialized

STATUS_SUCCESS

Example:	Uninitialize1089 ( 0 );

# GetRevisions1089                                                                                                               _

Function:	Retrieve the revision of the hardware device, the DLL, and the device driver.

Syntax:	long GetRevisions1089( long lngDevice, long *plngRevDevice, double *pdblRevDLL, double *pdblRevDriver);

Parameters:	lngDevice – The device to control.

plngRevDevice – The pointer to the hardware device variable.

pdblRevDLL – The pointer to the DLL variable.

pdblRevDriver – The pointer to the driver variable.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	GetRevisions1089 ( 0, &lngRevDevice, &dblRevDLL, &dblRevDriver );

# ExecutePlay1089                                                                                                                _

Function:	Start streaming a HDTV video or continue streaming video after a ‘pause’.

Syntax:	long ExecutePlay1089(long lngDevice , LPCTSTR strFilename, long lngPercentStart, long lngPercentEnd, long lngProgram);

Parameters:	lngDevice – The device to control.

strFilename – The HDTV video path and name with extension.

lngPercentStart – The starting point of the video file.

lngPercentEnd – The ending point (percent) of the video file.

lngProgram – The program index to select in the video.

Note:	The program index should be set to 0 if the program index is not known (see GetProgramCountFile1089 to retrieve a video program count before or after streaming occurs).

To stream the whole video file set lngPercentStart = 0 and lngPercentEnd = 100.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

STATUS_PLAY_RANGE_INVALID

Example:	ExecutePlay1089 (0, “C:\Clip.trp”, 0, 100, 0 );

# ExecuteStop1089                                                                                                                _

Function:	Stop streaming a HDTV video.

Syntax:	long ExecuteStop1089(long lngDevice);

Parameters:	lngDevice – The device to control.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	ExecuteStop1089(0);

# ExecutePause1089                                                                                                              _

Function:	‘Pause’ streaming a HDTV video in the current location or continue streaming video after a ‘pause’.

Syntax:	long ExecutePause1089(long lngDevice);

Parameters:	lngDevice – The device to control.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	ExecutePause1089(0);

# GetProgramCount1089                                                                                                     _

Function:	Retrieve the program count and the program numbers of the currently streaming video.

Syntax:	long GetProgramCount1089(long lngDevice,  long * plngProgramInfo);

Parameters:	lngDevice – The device to control.

plngProgramInfo – A pointer to the variable to store the binary represetation of the files in the video.  The binary representation is as follows:  if bit N is a 1 then program N+1 is present (see below example).

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

The number of programs present in the stream.

Example:	GetProgramCount1089( 0, &lngProgramInfo );

# GetProgramCountFile1089                                                                                               _

Function:	Retrieve the program count and the program numbers from a file.

Syntax:	long GetProgramCountFile1089 (long lngDevice,  long * plngProgramInfo);

Parameters:	lngDevice – The device to control.

plngProgramInfo – A pointer to the variable to store the binary represetation of the files in the video (see GetProgramCount1089 for description).

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

On success the function returns the number of programs present in the stream.

Example:	GetProgramCountFile1089 ( 0, &lngProgramInfo );

# Set8VSBAdjustChannel1089                                                                                            _

Function:	Set the output channel for 8-VSB.

Syntax:	long Set8VSBAdjustChannel1089(long lngDevice,  long lngChannel);

Parameters:	lngDevice – The device to control.

lngChannel – The output channel for 8-VSB.  Range: 2 - 69

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

STATUS_CHANNEL_RANGE_ERROR

Example:	Set8VSBAdjustChannel1089 (0, 14);

# GetRevisionXilinx1089                                                                                                      _

Function:	Retrieve the revision of the Xilinx load (firmware).

Syntax:	long GetRevisionsXilinx1089(long lngDevice, double pdblRevXilinx , long plngBoardType);

Parameters:	lngDevice – The device to control.

pdblRevXilinx – The pointer to the Xilinx variable.

plngBoardType – The pointer to the Board Type variable.

Board type is as follows:	 0 = 300A, 1 = 300B,  2 = 400A

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	GetRevisionsXilinx1089( 0, &dblRevXilinx, &lngBoardType );

# OnProgressSetup1089                                                                                                        _

Function:	Setup the OnProgress callback function.

Syntax:	long OnProgressSetup1089 ( PFN_ON_PROGRESS OnProgress_);

Parameters:	OnProgress_ – The address of the function to callback to.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	OnProgressSetup1089 (OnProgress1089);

VB Example:	Call OnProgressSetup1089 (Addressof OnProgress1089)

# OnStatusSetup1089                                                                                                            _

Function:	Setup the OnStatus callback function.

Syntax:	long OnStatusSetup1089 (PFN_ON_PROGRESS OnStatus_);

Parameters:	OnStatus_ – The address of the function to callback to.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	OnStatusSetup1089 (OnStatus1089);

VB Example:	Call OnStatusSetup1089 (Addressof OnStatus1089)

# ExecutePlaySeamless1089                                                                                                 _

Function:	Start streaming a HDTV video or continue streaming video after a ‘pause’.

Syntax:	long ExecutePlaySeamless1089(long lngDevice , LPCTSTR strFilename, long lngNullPackets);

Parameters:	lngDevice – The device to control.

strFilename – The HDTV video path and name with extension.

LngNullPackets – Reserved.

Note:	1) Send this function to play a file in seamless mode.

2) When the status message returns with STATUS_PLAY_START get the next file in the playlist.

3) Go to step 1.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	ExecutePlaySeamless1089 (0, “C:\Clip1.trp”, 0);

# GetCurrentlyPlayingFile1089                                                                                           _

Function:	Start Get the currently playing filename.

Syntax:	long GetCurrentPlayingFile1089 (long lngDevice , LPCTSTR strFilename);

Parameters:	lngDevice – The device to control.

strFilename – The passed out string for the filename.

Return Value:	STATUS_FAILURE – Hardware not initialized or DLL failure

STATUS_SUCCESS

Example:	GetCurrentPlayingFile1089 (0, strFilenamePlaceholder );

# RestampStream1089                                                                                                   _

Function:	Enable/Disable hardware restamping (calculate and overwrite PCR information in the stream).

Syntax:	long RestampStream1089(long lngDevice , long lMode);

Parameters:	lngDevice – The device to control.

lMode – 	0 – Disable firmware restamping.

1 – Enable firmware restamping

Return Value:	STATUS_FAILURE – Hardware not initialized, DLL failure, or incorrect mode.

STATUS_SUCCESS

Example:	RestampStream1089 (0, 1);

# SetPids1089                                                                                                   _

Function:	Set the PIDs for the seamless playback and restamping.

Syntax:	long SetPids1089(long lngDevice , long lPmtPid, long lVidPid, long lAudPid, long lPcrPid);

Parameters:	lngDevice – The device to control.

lPmtPid – The PMT PID.

lVidPid – The video PID

lAudPid – The audio PID

lPcrPid – The PCR PID

Return Value:	STATUS_FAILURE – Hardware not initialized, DLL failure, or a PID > 0x1FFF.

STATUS_SUCCESS

Example:	SetPids1089 (0, 1);

# Board1089_ Set8VsbVct                                                                                                    _

Function:	Set 8VSB frequency, VCT major_channel_number and minor_channel_number, and VCT select (Cable or Terrestrial).

Syntax:	long Board1089_Set8VsbVct(long lngDevice ,double freq, int Major,int Minor, int Mode);

Parameters:	lngDevice – The device to control.

freq – The frequency of 8VSB output.

major – The major channel number (1-999).

minor – The minor channel number (0-999).

mode – Table selection TVCT or CVCT (default: TVCT)

0 – TVCT, 1 - CVCT

Note: 		Range: 470 MHz – 520 MHz (Channels 14 – 21)

Note: 		Resolution: 100 KHz

Return Value:	STATUS_FAILURE – Hardware not initialized, DLL failure, or out of frequency range.

STATUS_SUCCESS

Example:	Board1089_Set8VsbVct (0, 473000000, 66, 1, 0);

# Board1089_ SetVct                                                                                                      _

Function:	VCT major_channel_number and minor_channel_number, and VCT select (Cable or Terrestrial).

Syntax:	long Board1089_SetVct(long lngDevice , int Major,int Minor, int Mode);

Parameters:	lngDevice – The device to control.

major – The major channel number (1-999).

minor – The minor channel number (0-999).

mode – Table selection TVCT or CVCT (default: TVCT)

0 – TVCT, 1 - CVCT

Return Value:	STATUS_FAILURE – Hardware not initialized, DLL failure, or out of frequency range.

STATUS_SUCCESS

Example:	Board1089_SetVct (0, 66, 1, 0);

## Callbacks:

VB Warning: A thread created by CreateThread API function calls these VB Callbacks, so an object cannot be directly written to, but a variable can be.  The sample app has a timer and global variables to handle callback information from C++. See Microsoft document Q198607.

# OnProgress1089                                                                                                                 _

Function:	The callback function for the current percent (point) where the video is streaming data from.

Syntax:	void OnProgress1089 (long lngDevice, long lngPercent);

## VB Syntax:	Public Sub OnProgress1089 (ByVal lngDevice As Long, ByVal lngPercent As Long)

Parameters:	lngDevice – The device that is sending back the Progress message

lngPercent – The current percent (point) where the video is streaming data from

Return Value:	N/A

Example:	OnProgress1089 (0, 35);

# OnStatus1089                                                                                                                   _

Function:	The callback function for a message being passed back to the app.

Syntax:	void OnStatus1089 (long lngDevice, long lngStatus);

VB Syntax:	Public Sub OnStatus1089 (ByVal lngDevice As Long, ByVal lngStatus As Long)

Parameters:	lngDevice – The device that is sending back the Status message

lngStatus – The current message passed back (see ENUM_STATUS enumeration).

Note:	STATUS_PLAY_START - the start of the file being played STATUS_PLAY_COMPLETE - the end of the file

Return Value:	N/A

Example:	OnStatus1089 (0, STATUS_PLAY_START);